3D printing can be integrated into nearly every stage of making paint-your-own-pottery (PYOP) pieces, most commonly to create master models, produce plaster slip-casting molds, or print the clay body directly.

## 1. 3D Printing Master Models (The Most Common Method)

Standard desktop 3D printers (FDM or SLA resin) are widely used to sculpt and iterate physical prototypes:

- Digital Design (CAD): Custom figurines, mugs, or ornaments are modeled digitally in programs like Blender or Fusion 360.

- Positive Model: The model is printed in plastic or resin, sanded smooth, and sealed.

- Plaster Casting: Plaster is poured directly over the 3D print to produce multi-part production molds for traditional slip casting.

## 2. Direct 3D Printing of Molds

Instead of casting plaster over a physical master, printers can manufacture the tooling directly:

- Rigid Resin/Filament Shells: Print multi-part exterior mold jackets that snap together for casting liquid plaster or slip.

- Flexible TPU/Silicone Inserts: Flexible 3D-printed filaments allow for complex undercuts that would normally lock up rigid plaster molds.

- Porous Binder Jetting: Industrial binder-jet printers can print porous plaster-like tooling directly from CAD files without manual mold-making.

## 3. Direct Ceramic 3D Printing (Paste Extrusion)

Pottery can be printed directly using clay instead of plastic:

- Liquid Clay Extrusion: Clay printers (e.g., WASP, Lutum, 3D Potter) extrude wet ceramic paste layer-by-layer directly onto a build plate.

- Custom Geometric Forms: Enables hollow geometries, parametric lattice walls, and twisted internal structures that cannot be demolded using slip casting.

- Post-Processing: Once printed, the greenware dries to a bone-dry state and is bisque-fired in a kiln exactly like traditional slip-cast ware.

## Comparison: Traditional Slip Casting vs. 3D-Assisted Methods

| Method | Best For | Production Speed | Unit Cost |
| --- | --- | --- | --- |
| Traditional Slip Casting | Standard PYOP catalog items (plates, simple mugs) | Very high (once molds exist) | Lowest |
| 3D Master → Plaster Mold | Custom brand mascots, personalized batches, local studio exclusives | High for production runs | Low to Moderate |
| Direct Ceramic 3D Printing | One-off bespoke items, complex lattices, on-demand personalized shapes | Low (print-on-demand) | Highest |

If you'd like to explore this further, I can:

- Detail the best 3D modeling workflows for creating slip-castable multi-part molds (draft angles, parting lines).

- Compare entry-level ceramic 3D printers and their clay paste requirements.

- Explain how shrinkage rates during bisque firing impact 3D print dimensions.